VSOLUTION PON APPLICATION WHITE PAPER—EPON APP NOTE/EXAMPLE

Version V1.0.1

Release Date 2016-06-13

Guangzhou V-SOLUTION Technology Co., Ltd

Contents

Case 1 SFU Bridge Without VLAN	4

Network Diagram	4

Planning Data	4

Configuring the OLT	5

Configuring the ONU	5

Case 2 SFU Bridge With VLAN	6

Network Diagram	6

Planning Data	6

Configuring the OLT	7

Configuring the ONU	9

Case 3 SFU Bridge With VLAN--IPTV	11

Network Diagram	11

Planning Data	11

Configuring the OLT	12

Configuring the ONU	15

Case 4 SFU Route Without VLAN	18

Network Diagram	18

Planning Data	18

Configuring the OLT	20

Configuring the ONU	20

Case 5 SFU Router With VLAN	25

Network Diagram	25

Planning Data	25

Configuring the OLT	27

Configuring the ONU	29

Case 6 HGU Router With VLAN--Internet	34

Network Diagram	34

Planning Data	34

Configuring the OLT	36

Configuring the ONU	37

Case 7 HGU Router With VLAN--VoIP	42

Network Diagram	42

Planning Data	42

Configuring the OLT	43

Configuring the ONU	45

Case 8 HGU Bridge With VLAN--IPTV	50

Network Diagram	50

Planning Data	50

Configuring the OLT	51

Configuring the ONU	55

Case 9 HGU Router/Bridge With VLAN Internet--VoIP--IPTV	57

Network Diagram	57

Planning Data	57

Configuring the OLT	60

Configuring the ONU	60

Case 1 SFU Bridge Without VLAN

DHCP/PPPoE/Static

Network Diagram

10.1.1.1/24

Planning Data

| Item |  | Description | Example |
| --- | --- | --- | --- |
| ONU Information | Slot No. | The number of the slot actually used. | 0 |
|  | PON No. | The number of the actually used PON port. | 1 |
|  | ONU Auth No. | Configure according to the network planning of the operator. | 1 |
|  | ONU Type | The type of the ONU. | 2801HE |
| OLT | Service Uplink VLAN | The VLAN ID of the uplink port service. | Default (VLAN = 1 UNTAG PVID = 1) |
|  | Service Uplink port | Configure according to the number of the actually used uplink port. | GE8 |
|  | Service PON VLAN | The VLAN ID of the PON port service. | Default (VLAN = 1 UNTAG PVID = 1) |
|  | Service PON port | Configure according to the number of the actually used PON port. | PON1 |
| ONU | Service LAN VLAN | The VLAN ID of the LAN port service. | Default (VLAN mode = TRANSPARENT) |
|  | Service LAN port | The number of the actually used ONU port. | LAN1 |
| PC | If DHCP mode | The router use DHCP assign ip to PC. | Gateway = 10.1.1.1 IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |
|  | If PPPoE mode | The router use PPPoE assign ip to PC. | IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 User name = test Password = test |
|  | If Static mode | The router use Static assign ip to PC. | Gateway = 10.1.1.1 IP = 10.1.1.100 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |

Configuring the OLT

Default Configuration

Configuring the ONU

Default Configuration

Case 2 SFU Bridge With VLAN

DHCP/PPPoE/Static

Network Diagram

10.1.1.1/24

Planning Data

| Item |  | Description | Example |
| --- | --- | --- | --- |
| ONU Information | Slot No. | The number of the slot actually used. | 0 |
|  | PON No. | The number of the actually used PON port. | 1 |
|  | ONU Auth No. | Configure according to the network planning of the operator. | 1 |
|  | ONU Type | The type of the ONU. | 2801HE |
| OLT | Service Uplink VLAN | The VLAN ID of the uplink port service. | VLAN mode = UNTAG PVID = 100 |
|  | Service Uplink port | Configure according to the number of the actually used uplink port. | GE8 |
|  | Service PON VLAN | The VLAN ID of the PON port service. | VLAN mode = TAG VLAN ID = 100 |
|  | Service PON port | Configure according to the number of the actually used PON port. | PON1 |
| ONU | Service LAN VLAN | The VLAN ID of the LAN port service.(CTC VLAN mode) | VLAN mode = TAG PVID = 100 |
|  | Service LAN port | The number of the actually used ONU port. | LAN1 |
| PC | If DHCP mode | The router use DHCP assign ip to PC. | Gateway = 10.1.1.1 IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |
|  | If PPPoE mode | The router use PPPoE assign ip to PC. | IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 User name = test Password = test |
|  | If Static mode | The router use Static assign ip to PC. | Gateway = 10.1.1.1 IP = 10.1.1.100 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |

Configuring the OLT

CLI

epon-olt(config)# vlan 100

epon-olt(config-vlan-100)# exit

epon-olt(config)# interface gigabitethernet 0/8

epon-olt(config-if-ge0/8)# switchport hybrid vlan 100 untagged

epon-olt(config-if-ge0/8)# switchport hybrid pvid vlan 100

epon-olt(config-if-ge0/8)# exit

epon-olt(config)# interface epon 0/1

epon-olt(config-pon-0/1)# switchport hybrid vlan 100 tagged

epon-olt(config-pon-0/1)# exit

EMS

1. Right-click the OLT icon and select Configuration->VLAN Configuration

2. Click the Add button, configure according to the planning data.

3. Click the commit button to complete the configuration.

4. Right-click the OLT icon and select Configuration->Port Configuration

Configure according to the planning data.

5. Click the Apply button to complete the configuration.

Configuring the ONU

CLI

epon-olt(config)# interface epon  0/1

epon-olt(config-pon-0/1)# onu 1 ctc eth 1 vlan mode tag

epon-olt(config-pon-0/1)# onu 1 ctc eth 1 vlan pvid 100 pri 3

epon-olt(config-pon-0/1)# exit

EMS

1. Right-click the ONU icon and select Configuration->ONU Port Configuration

2. Choose VLAN Configuration->port 1, configure according to the planning data.

3. Click the Save button to complete the configuration.

Case 3 SFU Bridge With VLAN--IPTV

Network Diagram

10.1.3.1/24

DHCP/PPPoE/Static

Planning Data

| Item |  | Description | Example |
| --- | --- | --- | --- |
| ONU Information | Slot No. | The number of the slot actually used. | 0 |
|  | PON No. | The number of the actually used PON port. | 1 |
|  | ONU Auth No. | Configure according to the network planning of the operator. | 1 |
|  | ONU Type | The type of the ONU. | 2801HE |
| OLT | Service Uplink VLAN | The VLAN ID of the uplink port service. | VLAN mode = UNTAG PVID = 100 |
|  | Service Uplink port | Configure according to the number of the actually used uplink port. | GE8 |
|  | Service PON VLAN | The VLAN ID of the PON port service. | VLAN mode = TAG VLAN ID = 100 |
|  | Service PON port | Configure according to the number of the actually used PON port. | PON1 |
| ONU | Service LAN VLAN | The VLAN ID of the LAN port service.(CTC VLAN mode) | VLAN mode = TAG PVID = 100 |
|  | Service LAN port | The number of the actually used ONU port. | LAN1 |
| STB | Management IP | The IP for STB. | Gateway = 10.1.3.1 IP = 10.1.3.100 IP Mask = 255.255.255.0 |

Configuring the OLT

CLI

epon-olt(config)# vlan 100

epon-olt(config-vlan-100)# exit

epon-olt(config)# interface gigabitethernet 0/8

epon-olt(config-if-ge0/8)# switchport hybrid vlan 100 untagged

epon-olt(config-if-ge0/8)# switchport hybrid pvid vlan 100

epon-olt(config-if-ge0/8)# exit

epon-olt(config)# interface epon 0/1

epon-olt(config-pon-0/1)# switchport hybrid vlan 100 tagged

epon-olt(config-pon-0/1)# exit

epon-olt(config)# ip igmp snooping enable

epon-olt(config)# ip igmp snooping general-query-packet enable

epon-olt(config)# ip igmp snooping mrouter vlan 100 interface gigabitethernet 0/8

epon-olt(config)# interface epon 0/1

epon-olt(config-pon-0/1)# ip igmp snooping user-vlan 100 group-vlan 100 tagged

epon-olt(config-pon-0/1)# exit

EMS

1. Right-click the OLT icon and select Configuration->VLAN Configuration

2. Click the Add button, configure according to the planning data.

3. Click the commit button to complete the configuration.

4. Right-click the OLT icon and select Configuration->Port Configuration

Configure according to the planning data.

5. Click the Apply button to complete the configuration.

6. Right-click the OLT icon and select Configuration->IGMP Configuration

configure according to the planning data.

7. Choose Basic Configuration, tick the IGMP Snooping and General Query Packet , click the Apply button to complete the configuration.

8. Choose Router Configuration, click the Add button to create a item, then modify the parameters according to the planning data.

Click the Save button to complete the configuration.

9. Choose User Vlan Configuration, click the Add button to create a item, then modify the parameters according to the planning data.

Click the Save button to complete the configuration.

Configuring the ONU

CLI

epon-olt (config)# inter epon 0/1

epon-olt (config-pon-0/1)# onu 1 ctc eth 1 vlan mode tag

epon-olt (config-pon-0/1)# onu 1 ctc eth 1 vlan pvid 100 pri 0

epon-olt (config-pon-0/1)# onu 1 ctc eth 1 mc_vlan add 100

epon-olt (config-pon-0/1)# onu 1 ctc eth 1 mc_tagstrip enable

EMS

1. Right-click the ONU icon and select Configuration->ONU Port Configuration

2. Choose VLAN Configuration->port 1, configure according to the planning data.

3. Click the Save button to complete the configuration.

4. Choose IGMP Configuration->IGMP VLAN Configuration->port 1,click Add, configure according to the planning data.

5. Click the Save button to complete the configuration.

6. Choose Port IGMP VLAN ->port 1,click Add, configure according to the planning data.

Case 4 SFU Route Without VLAN

DHCP/PPPoE/Static

Network Diagram

10.1.1.1/24

Planning Data

| Item |  | Description | Example |
| --- | --- | --- | --- |
| ONU Information | Slot No. | The number of the slot actually used. | 0 |
|  | PON No. | The number of the actually used PON port. | 1 |
|  | ONU Auth No. | Configure according to the network planning of the operator. | 1 |
|  | ONU Type | The type of the ONU. | 2801HW |
| OLT | Service Uplink VLAN | The VLAN ID of the uplink port service. | Default (VLAN = 1 UNTAG PVID = 1) |
|  | Service Uplink port | Configure according to the number of the actually used uplink port. | GE8 |
|  | Service PON VLAN | The VLAN ID of the PON port service. | Default (VLAN = 1 UNTAG PVID = 1) |
|  | Service PON port | Configure according to the number of the actually used PON port. | PON1 |
| ONU | Service LAN VLAN | The VLAN ID of the LAN port service. | Default No VLAN |
|  | Service LAN port | The number of the actually used ONU port. | LAN1 |
|  | WiFi | Wireless’s SSID name,Security. | SSID1 name = SSID-1 Security method = WAP2 Mixed WPA Encryption = TKIP WPA2 Encryption = AES KEY = 12345678 |
|  | Service WAN VLAN | The VLAN ID of the PON(WAN) port service. | Default No VLAN |
|  | If WAN use DHCP mode | The router use DHCP assign ip to ONU. | Gateway = 10.1.1.1 IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |
|  | If WAN use PPPoE mode | The router use PPPoE assign ip to ONU. | IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 User name = test Password = test |
|  | If WAN use Static mode | The router use Static assign ip to ONU. | Gateway = 10.1.1.1 IP = 10.1.1.100 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |
| PC | DHCP mode | The ONU use DHCP assign ip to PC. | Gateway = 192.168.1.1 IP = 192.168.1.40 IP Mask = 255.255.255.0 DNS = 192.168.1.1 |

Configuring the OLT

Default Configuration

Configuring the ONU

ONU WEB

If DHCP mode

The connection status

If PPPoE mode

The connection status

If Static mode

The connection status

WiFi

Case 5 SFU Router With VLAN

DHCP/PPPoE/Static

Network Diagram

10.1.1.1/24

Planning Data

| Item |  | Description | Example |
| --- | --- | --- | --- |
| ONU Information | Slot No. | The number of the slot actually used. | 0 |
|  | PON No. | The number of the actually used PON port. | 1 |
|  | ONU Auth No. | Configure according to the network planning of the operator. | 1 |
|  | ONU Type | The type of the ONU. | 2801HW |
| OLT | Service Uplink VLAN | The VLAN ID of the uplink port service. | VLAN mode = UNTAG PVID = 100 |
|  | Service Uplink port | Configure according to the number of the actually used uplink port. | GE8 |
|  | Service PON VLAN | The VLAN ID of the PON port service. | VLAN mode = TAG VLAN ID = 100 |
|  | Service PON port | Configure according to the number of the actually used PON port. | PON1 |
| ONU | Service LAN VLAN | The VLAN ID of the LAN port service. | Default No VLAN |
|  | Service LAN port | The number of the actually used ONU port. | LAN1 |
|  | WiFi | Wireless’s SSID name,Security. | SSID1 name = SSID-1 Security method = WAP2 Mixed WPA Encryption = TKIP WPA2 Encryption = AES KEY = 12345678 |
|  | Service WAN VLAN | The VLAN ID of the PON(WAN) port service. | VLAN ID = 100 |
|  | If WAN use DHCP mode | The router use DHCP assign ip to ONU. | Gateway = 10.1.1.1 IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |
|  | If WAN use PPPoE mode | The router use PPPoE assign ip to ONU. | IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 User name = test Password = test |
|  | If WAN use Static mode | The router use Static assign ip to ONU. | Gateway = 10.1.1.1 IP = 10.1.1.100 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |
| PC | DHCP mode | The ONU use DHCP assign ip to PC. | Gateway = 192.168.1.1 IP = 192.168.1.40 IP Mask = 255.255.255.0 DNS = 192.168.1.1 |

Configuring the OLT

CLI

epon-olt(config)# vlan 100

epon-olt(config-vlan-100)# exit

epon-olt(config)# interface gigabitethernet 0/8

epon-olt(config-if-ge0/8)# switchport hybrid vlan 100 untagged

epon-olt(config-if-ge0/8)# switchport hybrid pvid vlan 100

epon-olt(config-if-ge0/8)# exit

epon-olt(config)# interface epon 0/1

epon-olt(config-pon-0/1)# switchport hybrid vlan 100 tagged

epon-olt(config-pon-0/1)# exit

EMS

1. Right-click the OLT icon and select Configuration->VLAN Configuration

2. Click the Add button, configure according to the planning data.

3. Click the commit button to complete the configuration.

4. Right-click the OLT icon and select Configuration->Port Configuration

configure according to the planning data.

5. Click the Apply button to complete the configuration.

Configuring the ONU

ONU WEB

If DHCP mode

The connection status

If PPPoE mode

The connection status

If Static mode

The connection status

WiFi

Case 6 HGU Router With VLAN--Internet

DHCP/PPPoE/Static

Network Diagram

10.1.1.1/24

Planning Data

| Item |  | Description | Example |
| --- | --- | --- | --- |
| ONU Information | Slot No. | The number of the slot actually used. | 0 |
|  | PON No. | The number of the actually used PON port. | 1 |
|  | ONU Auth No. | Configure according to the network planning of the operator. | 1 |
|  | ONU Type | The type of the ONU. | HG326UEG |
| OLT | Service Uplink VLAN | The VLAN ID of the uplink port service. | VLAN mode = UNTAG PVID = 100 |
|  | Service Uplink port | Configure according to the number of the actually used uplink port. | GE8 |
|  | Service PON VLAN | The VLAN ID of the PON port service. | VLAN mode = TAG VLAN ID = 100 |
|  | Service PON port | Configure according to the number of the actually used PON port. | PON1 |
| ONU | Service LAN VLAN | The VLAN ID of the LAN port service. | Default No VLAN |
|  | Service LAN port | The number of the actually used ONU port. | LAN1 |
|  | WiFi | Wireless’s SSID name,Security. | SSID1 name = SSID-1 Security method = WAP2 Mixed WPA Encryption = TKIP WPA2 Encryption = AES KEY = 12345678 |
|  | Service WAN VLAN | The VLAN ID of the PON(WAN) port service. | VLAN ID = 100 |
|  | If WAN use DHCP mode | The router use DHCP assign ip to terminal user. | Gateway = 10.1.1.1 IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |
|  | If WAN use PPPoE mode | The router use PPPoE assign ip to terminal user. | IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 User name = test Password = test |
|  | If WAN use Static mode | The router use Static assign ip to terminal user. | Gateway = 10.1.1.1 IP = 10.1.1.100 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |
| PC | DHCP mode | The ONU use DHCP assign ip to PC. | Gateway = 192.168.1.1 IP = 192.168.1.2 IP Mask = 255.255.255.0 DNS = 192.168.1.1 |

Configuring the OLT

CLI

epon-olt(config)# vlan 100

epon-olt(config-vlan-100)# exit

epon-olt(config)# interface gigabitethernet 0/8

epon-olt(config-if-ge0/8)# switchport hybrid vlan 100 untagged

epon-olt(config-if-ge0/8)# switchport hybrid pvid vlan 100

epon-olt(config-if-ge0/8)# exit

epon-olt(config)# interface epon 0/1

epon-olt(config-pon-0/1)# switchport hybrid vlan 100 tagged

epon-olt(config-pon-0/1)# exit

EMS

1. Right-click the OLT icon and select Configuration->VLAN Configuration

2. Click the Add button, configure according to the planning data.

3. Click the commit button to complete the configuration.

4. Right-click the OLT icon and select Configuration->Port Configuration

configure according to the planning data.

5. Click the Apply button to complete the configuration.

Configuring the ONU

ONU WEB

If DHCP mode

The connection status

If PPPoE mode

The connection status

If Static mode

The connection status

WiFi

Case 7 HGU Router With VLAN--VoIP

DHCP/PPPoE/Static

Network Diagram

10.1.2.1/24

Planning Data

| Item |  | Description | Example |
| --- | --- | --- | --- |
| ONU Information | Slot No. | The number of the slot actually used. | 0 |
|  | PON No. | The number of the actually used PON port. | 1 |
|  | ONU Auth No. | Configure according to the network planning of the operator. | 1 |
|  | ONU Type | The type of the ONU. | HG326UEG |
| OLT | Service Uplink VLAN | The VLAN ID of the uplink port service. | VLAN mode = UNTAG PVID = 200 |
|  | Service Uplink port | Configure according to the number of the actually used uplink port. | GE6 |
|  | Service PON VLAN | The VLAN ID of the PON port service. | VLAN mode = TAG VLAN ID = 200 |
|  | Service PON port | Configure according to the number of the actually used PON port. | PON1 |
| ONU | Service WAN VLAN | The VLAN ID of the PON(WAN) port service. | VLAN ID = 200 |
|  | Service POTS port | The number of the actually used ONU port. | POTS1 |
|  | If WAN use DHCP mode | The router use DHCP assign ip to ONU. | Gateway = 10.1.2.1 IP Range 10.1.2.2-10.1.2.254 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |
|  | If WAN use PPPoE mode | The router use PPPoE assign ip to ONU. | IP Range 10.1.2.2-10.1.2.254 IP Mask = 255.255.255.0 User name = test1 Password = test1 |
|  | If WAN use Static mode | The router use Static assign ip to ONU. | Gateway = 10.1.2.1 IP = 10.1.2.100 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |

Configuring the OLT

CLI

epon-olt(config)# vlan 200

epon-olt(config-vlan-200)# exit

epon-olt(config)# interface gigabitethernet 0/6

epon-olt(config-if-ge0/6)# switchport hybrid vlan 200 untagged

epon-olt(config-if-ge0/6)# switchport hybrid pvid vlan 200

epon-olt(config-if-ge0/6)# exit

epon-olt(config)# interface epon 0/1

epon-olt(config-pon-0/1)# switchport hybrid vlan 200 tagged

epon-olt(config-pon-0/1)# exit

EMS

1. Right-click the OLT icon and select Configuration->VLAN Configuration

2. Click the Add button, configure according to the planning data.

3. Click the commit button to complete the configuration.

4. Right-click the OLT icon and select Configuration->Port Configuration

configure according to the planning data.

5. Click the Apply button to complete the configuration.

Configuring the ONU

ONU WEB

If DHCP mode

The connection status

If PPPoE mode

The connection status

If Static mode

The connection status

VoIP

Case 8 HGU Bridge With VLAN--IPTV

DHCP/PPPoE/Static

Network Diagram

10.1.3.1/24

Planning Data

| Item |  | Description | Example |
| --- | --- | --- | --- |
| ONU Information | Slot No. | The number of the slot actually used. | 0 |
|  | PON No. | The number of the actually used PON port. | 1 |
|  | ONU Auth No. | Configure according to the network planning of the operator. | 1 |
|  | ONU Type | The type of the ONU. | HG326UEG |
| OLT | Service Uplink VLAN | The VLAN ID of the uplink port service. | VLAN mode = UNTAG PVID = 300 |
|  | Service Uplink port | Configure according to the number of the actually used uplink port. | GE7 |
|  | Service PON VLAN | The VLAN ID of the PON port service. | VLAN mode = TAG VLAN ID = 300 |
|  | Service PON port | Configure according to the number of the actually used PON port. | PON1 |
| ONU | Service WAN VLAN | The VLAN ID of the PON(WAN) port service. | VLAN mode= Transparent |
|  | Service LAN VLAN | The VLAN ID of the LAN port service. | VLAN mode = UNTAG LAN VLAN ID = 300 Multicast VLAN ID = 300 |
|  | Service LAN port | The number of the actually used ONU port. | LAN4 |
| STB | Management IP | The IP for STB. | Gateway = 10.1.3.1 IP = 10.1.3.100 IP Mask = 255.255.255.0 |

Configuring the OLT

CLI

epon-olt(config)# vlan 300

epon-olt(config-vlan-300)# exit

epon-olt(config)# interface gigabitethernet 0/7

epon-olt(config-if-ge0/7)# switchport hybrid vlan 300 untagged

epon-olt(config-if-ge0/7)# switchport hybrid pvid vlan 300

epon-olt(config-if-ge0/7)# exit

epon-olt(config)# interface epon 0/1

epon-olt(config-pon-0/1)# switchport hybrid vlan 300 tagged

epon-olt(config-pon-0/1)# exit

epon-olt(config)# ip igmp snooping enable

epon-olt(config)# ip igmp snooping general-query-packet enable

epon-olt(config)# ip igmp snooping mrouter vlan 300 interface gigabitethernet 0/7

epon-olt(config)# interface epon 0/1

epon-olt(config-pon-0/1)# ip igmp snooping user-vlan 300 group-vlan 300 tagged

epon-olt(config-pon-0/1)# exit

EMS

1. Right-click the OLT icon and select Configuration->VLAN Configuration

2. Click the Add button, configure according to the planning data.

3. Click the commit button to complete the configuration.

4. Right-click the OLT icon and select Configuration->Port Configuration

configure according to the planning data.

5. Click the Apply button to complete the configuration.

6. Right-click the OLT icon and select Configuration->IGMP Configuration

configure according to the planning data.

7. Choose Basic Configuration, tick the IGMP Snooping and General Query Packet , click the Apply button to complete the configuration.

8. Choose Router Configuration, click the Add button to create a item, then modify the parameters according to the planning data.

Click the Save button to complete the configuration.

9. Choose User Vlan Configuration, click the Add button to create a item, then modify the parameters according to the planning data.

Click the Save button to complete the configuration.

Configuring the ONU

ONU WEB

IPTV(LAN and Multicast VLAN)

Case 9 HGU Router/Bridge With VLAN Internet--VoIP--IPTV

10.1.3.1/24

Network Diagram

10.1.2.1/24

10.1.1.1/24

DHCP/PPPoE/Static

DHCP/PPPoE/Static

Planning Data

| Item |  | Description | Example |
| --- | --- | --- | --- |
| ONU Information | Slot No. | The number of the slot actually used. | 0 |
|  | PON No. | The number of the actually used PON port. | 1 |
|  | ONU Auth No. | Configure according to the network planning of the operator. | 1 |
|  | ONU Type | The type of the ONU. | HG326UEG |
| OLT | Service Uplink VLAN | The VLAN ID of the uplink port service. | VLAN mode = UNTAG GE7,PVID = 300 for IPTV GE5,PVID = 200 for VoIP GE8,PVID = 100 for Internet |
|  | Service Uplink port | Configure according to the number of the actually used uplink port. | GE7, for IPTV GE5, for VoIP GE8, for Internet |
|  | Service PON VLAN | The VLAN ID of the PON port service. | VLAN mode = TAG VLAN ID = 300,200,100 |
|  | Service PON port | Configure according to the number of the actually used PON port. | PON1 |
| ONU | Service WAN VLAN | The VLAN ID of the PON(WAN) port service. | WAN1(for IPTV): VLAN mode= Transparent; WAN2(for VoIP): VLAN ID = 200; WAN3(for Internet): VLAN ID = 100; |
|  | Service LAN VLAN | The VLAN ID of the LAN port service. | LAN4(for IPTV): VLAN mode = UNTAG LAN VLAN ID = 300 Multicast VLAN ID = 300; LAN1(for Internet): Default |
|  | WiFi | Wireless’s SSID name,Security. | SSID1 name = SSID-1 Security method = WAP2 Mixed WPA Encryption = TKIP WPA2 Encryption = AES KEY = 12345678 |
|  | Service POTS port | The number of the actually used ONU port. | POTS1 |
|  | If WAN use DHCP mode | The router use DHCP assign ip to ONU. | Internet: Gateway = 10.1.1.1 IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 DNS = 8.8.8.8 VoIP: Gateway = 10.1.2.1 IP Range 10.1.2.2-10.1.1.254 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |
|  | If WAN use PPPoE mode | The router use PPPoE assign ip to ONU. | Internet: IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 User name = test Password = test VoIP: IP Range 10.1.2.2-10.1.2.254 IP Mask = 255.255.255.0 User name = test1 Password = test1 |
|  | If WAN use Static mode | The router use Static assign ip to ONU. | Internet: Gateway = 10.1.1.1 IP Mask = 255.255.255.0 VoIP: Gateway = 10.1.2.1 IP Mask = 255.255.255.0 |
| PC | DHCP mode | The ONU use DHCP assign ip to PC. | Gateway = 192.168.1.1 IP = 192.168.1.2 IP Mask = 255.255.255.0 DNS = 192.168.1.1 |
| STB | Management IP | The IP for STB. | Gateway = 10.1.3.1 IP = 10.1.3.100 IP Mask = 255.255.255.0 |

Configuring the OLT

CLI

As show in Case 5 to 7.

EMS

As show in Case 5 to 7.

Configuring the ONU

ONU WEB

As show in Case 5 to 7.